Ag Silver Testing: Using Amber Spot on testing solution

- Use a magnet to test the item. If it sticks to the magnet, it is NOT Silver.

- If it doesn’t stick to the magnet, look for a hallmark. (925)(.800)(.500)

- No hallmark? Then you need to do an acid test. First file a notch into the piece to be tested. This will remove any plating and reveal the core of the piece.

- Now apply one drop of acid from the bottle.

- Sterling Silver (925) will stain from amber to a deep Blood Red very quickly. (1-3 seconds)
lower puritys of silver will turn from amber to blood red over a longer period of time, example .500 coins will turn red in approx. (5-10 seconds)

- Not Silver will turn green and froth.(Or Green & Brown & Froth)

- No Reaction at all is a sign of not silver, this may indicate aluminium or stainless steel. (stainless steel is not magnetic)

This is a colour changing indicator test.
Amber = Not silver

Blood Red = Silver

Green/brown = Base metals Eg Copper

Material Safety Data Sheet Silver testing acid 10ml Bottle

1. Product name:   Spot On Acid Silver testing Solution (Amber Fluid)

| Nitric Acid above 10% & Below 50% HNO3 | Potassium dichromate K2Cr2O7 Below 15% |
| --- | --- |

2. Composition/information on ingredients

EC-No.: 231-714-2
EC Number: 231-906-6

3. Hazards identification:   Causes severe burns.  Toxic if swallowed,

4. First aid measures

•Eye contact: Irrigate thoroughly with water for at least 10 minutes. Obtain medical attention.

•Inhalation: Remove from exposure, rest and keep warm. In severe cases, or if exposure has been great, obtain medical attention.

•Skin contact: Drench the skin thoroughly with water. Remove contaminated clothing and wash before re use. Obtain medical attention.

•Ingestion: Wash out mouth thoroughly with water and give plenty of water to drink. Obtain medical attention.

5. Fire-fighting measures

Special risks:   May ignite combustible material.  May evolve toxic fumes in fire.

6. Accidental release measures

Wear appropriate protective clothing. Inform others to keep at a safe distance.

Absorb on an inert absorbent, transfer to a suitable container and arrange removal by disposal company. Wash site of spillage thoroughly with detergent and water.

7. Handling and storage

Handling: Work under fume cupboard. Do not inhale substance. Avoid contact with skin and eyes. Immediately change contaminated clothing. Apply skin- protective barrier cream. Wash hands and face after working with substance.

Use appropriate containment to avoid environmental contamination.

Unsuitable working materials: metals.

Storage: Store at room temperature (15 to 25°C recommended).

Keep well closed and protected from direct sunlight and

moisture.  Store away from combustible materials.

8. Exposure controls/personal protection

Personal protective equipment: As appropriate to the situation and the quantity handled. Engineering methods to control or prevent exposure are

preferred. Methods could include process enclosure or mechanical ventilation.

• Respirator: Positive-pressure hood

• Ventilation: Fume cupboard

•Gloves: High grade PVC

•Eye Protection: Goggles or face-shield

•Other Precautions: Plastic apron, sleeves, boots - if handling large quantities

.

9. Physical and chemical properties

Physical State:    Liquid

Appearance: Amber/Red

Odour:   Pungent

10. Stability and reactivity
Discoloration during storage. 
Substances to be avoided : Organic combustible substances, oxidisable substances, organic solvents, alcohols, ketones, aldehydes, amines, anilines, nitriles, organic nitro compounds, hydrazine and derivatives, acetylidene, metals, metal alloys, metallic oxides, alkali metals, alkaline earth metals, ammonia, bases acids, hydrides,

11. Toxicological information

Strongly corrosive substance.

•After skin contact: Burns.

•After eye contact: Burns. Risk of blindness!

•After inhalation of vapours: coughing, dyspnoea. Inhalation may lead to the formation of oedemas in the respiratory tract.

•After ingestion: tissue damage (mouth, oesophagus, gastrointestinal tract), severe pain (risk of perforation!), bloody vomiting, death.

12. Ecological information

Adverse ecological effects cannot be excluded in the event of improper handling or disposal. Harmful effect due to pH shift.

Further data: The following applies to nitrates in general: May contribute to the eutrophication of water supplies. Hazard for drinking water.

LC50 >500 mg/l, Fish Do not allow to enter drinking water supplies, waste water, or soil! Harmful effect due to pH shift.

13. Disposal considerations

Chemical residues are generally classified as special waste, and as such are covered by regulations which vary according to location.  Contact your local waste disposal authority for advice, or pass to a chemical disposal company. Rinse out empty containers thoroughly before returning for recycling.

14. Transport information

UN-No.:          2031 / 1463                 IMDG class: 8   / 5.1

IMO:               2/2031  / 5.1/1463             Packaging group: II

IATA:              2031 / 1463                Packaging group: II

15. Regulatory information

Labelling according to EC directives

Symbol: C     Corrosive.

R-phrases: R35 / - 49 - 8 - 25 -43Causes severe burns.

S-phrases S1 - 3 - 53 - 45 - 23 - 26 - 36 - 37- 39

Do not breathe fumes. In case of contact with eyes, rinse immediately with plenty of water and seek medical advice. Wear suitable protective clothing, gloves, and eye/face protection. In case of accident or if you feel unwell, seek medical advice immediately (show the label where possible).

16. Other information

Date of issue:    01/01/2019
Important Statement

The information above is believed to be accurate and represents the best information currently available from multiple crosschecked sources.  However, we make no warranty express or implied, with respect to such information, and we assume no liability resulting from its use.  Users should make their own investigations to determine the suitability of the information for their particular purposes.

In no way shall Spot on Tests be liable for any claims, losses, or damages of any third party or for lost profits or any special, indirect, incidental, consequential or exemplary damages, howsoever arising, even if the company has been advised of the possibility of such damages.